|  | Spartan6 - DSP48A1 |
| --- | --- |
|  | Supervisor: Eng. Kareem Wassem Created by: Abdelrahman Moamen Eltaweel 7/28/25 |

Table of Contents

DSP48A1 Slice Overview	4

Description	4

Key Features	4

Configuration and Usage	5

RTL code	5

Testbench code	11

Do file	17

QuestaSim Snippets	18

Verify Reset Operation	18

	Waveform	18

	Transcript	18

Verify DSP Path 1	19

	Waveform path 1	20

	Transcript path 1	20

Verify DSP Path 2	20

	Waveform path 2	21

	Transcript path 2	21

Verify DSP Path 3	22

	Waveform path 3	23

	Transcript path 3	23

Verify DSP Path 4	23

	Waveform path 4	24

	Transcript path 4	24

Constraint File	25

Elaboration	26

	Messages	26

	Schematic snippets	26

Synthesis	27

	Messages	27

	Utilization report	27

	Timing report	27

	Schematic snippets	28

Implementation	28

	Messages	28

	Utilization report	29

	Timing report	29

	Device snippets	29

Linting	30

	Snippets showing no errors	30

# Spartan6 - DSP48A1

# DSP48A1 Slice Overview

## Description

The Spartan-6 FPGA series by Xilinx is designed for a wide range of applications that require low power, cost-effectiveness, and high performance. The DSP48A1 slice is an integral part of the Spartan-6 FPGA family, specifically optimized for digital signal processing (DSP) tasks. This slice is a flexible and powerful arithmetic unit that can perform various operations like addition, subtraction, multiplication, and more complex functions, making it a cornerstone for implementing DSP algorithms efficiently.

## Key Features

- 16x16-bit Multiplier: Provides high-performance multiplication capability, essential for DSP algorithms. 
48-bit Accumulator: Allows for large dynamic range in accumulation operations, critical for applications like filtering and FFTs. 
Integrated Logic Functions: Supports various logical operations and shift functions, increasing the slice's versatility. 
Pipelining and Bypass Options: Enables high-speed data processing with options for pipelining or bypassing stages, depending on the application requirements. 
Low Power Consumption: Optimized for power efficiency, making it suitable for battery-operated and low-power designs. 
Configurable Input and Output Registers: Flexible configuration options to support different operational modes and data paths. 

Applications
Digital Signal Processing (DSP): Ideal for implementing high-performance DSP algorithms such as FIR filters, FFTs, and convolutions. 
Wireless Communication: Used in modulation, demodulation, and other signal processing tasks in wireless communication systems.
Audio and Video Processing: Facilitates real-time audio and video processing, including encoding, decoding, and filtering. 
Image Processing: Supports complex image processing operations like edge detection, filtering, and transformation. 
Medical Electronics: Utilized in medical imaging devices and diagnostic tools for real-time data processing.

## Configuration and Usage

The DSP48A1 slice can be configured in multiple modes to cater to various application needs. It can be used as a standalone arithmetic unit or as part of a larger, more complex DSP engine within the FPGA. Configuration involves setting up the slice through the Xilinx design tools (e.g., Vivado or ISE) to define the specific operations, pipelining, and data paths. The flexibility in configuration allows for tailored performance optimizations, enabling developers to balance power, speed, and resource utilization based on the application's requirements.

# RTL code

module dsp48a1

(

CLK, RSTA, RSTB, RSTC, RSTD, RSTM, RSTOPMODE, RSTP, RSTCARRYIN,

CEA, CEB, CEC, CECARRYIN, CED, CEM, CEOPMODE, CEP,

A, B, C, D, BCIN, PCIN, CARRYIN, OPMODE,

CARRYOUT, CARRYOUTF, BCOUT, M, PCOUT, P

);

//! Parameter (Attributes):

// The A0REG, A1REG, B0REG, and B1REG attributes can take values of 0 or 1.

// these values define the number of pipeline registers in the A & B input paths.

// A0REG defaults to 0 (no register). A1REG defaults to 1 (register). B0REG defaults to 0 (no register) B1REG defaults to 1 (register).

// A0 and B0 are the first stages of the pipelines.

// A1 and B1 are the second stages of the pipelines

parameter A0REG       = 1'b0;

parameter A1REG       = 1'b1;

parameter B0REG       = 1'b0;

parameter B1REG       = 1'b1;

// These attributes can take a value of 0 or 1.

// The number defines the number of pipeline stages. Default: 1 (registered)

parameter CREG        = 1'b1;

parameter DREG        = 1'b1;

parameter MREG        = 1'b1;

parameter PREG        = 1'b1;

parameter CARRYINREG  = 1'b1;

parameter CARRYOUTREG = 1'b1;

parameter OPMODEREG   = 1'b1;

// The CARRYINSEL attribute is used in the carry cascade input, either the CARRYIN input will be considered or the value of opcode[5].

// This attribute can be set to the string CARRYIN or OPMODE5.

// Default: OPMODE5. Tie the output of the mux to 0 if none of these string values exist.

parameter CARRYINSEL  = "OPMODE5";

// The B_INPUT attribute defines whether the input to the B port is routed from the B input (attribute = DIRECT) or the cascaded input

// (BCIN) from the previous DSP48A1 slice (attribute = CASCADE).

// Default: DIRECT. Tie the output of the mux to 0 if none of these string values exist.

parameter B_INPUT     = "DIRECT";

// The RSTTYPE attribute selects whether all resets for the DSP48A1 slice should have a synchronous or asynchronous reset capability.

// This attribute can be set to ASYNC or SYNC. Default: SYNC.

parameter RSTTYPE     = "SYNC";

// ! Data Ports:

input [17 : 0] A;  //18-bit data input to multiplier, and optionally to postadder

subtracter depending on the value of OPMODE[1:0].

input [17 : 0] B;    //18-bit data input to pre-adder/subtracter, to multiplier

depending on OPMODE[4], or to post-adder/subtracter depending

onOPMODE[1:0].

input [47 : 0] C;  //48-bit data input to post-adder/subtracter.

input [17 : 0] D;  //18-bit data input to pre-adder/subtracter. D[11:0] are

concatenated with A and B and optionally sent to post-

adder/subtracter depending on the value of OPMODE[1:0].

input [17 : 0] BCIN;

input CARRYIN;     //carry input to the post-adder/subtracter

output [35 : 0] M; //36-bit buffered multiplier data output, routable to the FPGA

logic. It either the output of the M register (MREG = 1) or

the direct output of the multiplier (MREG = 0).

output [47 : 0] P;  //Primary data output from the post-adder/subtracter. It is

either the output of the P register (PREG = 1) or the direct

بoutput of the postadder/subtracter (PREG = 0).

output CARRYOUT;   //Cascade carry out signal from post-adder/subtracter. It can

be registered in (CARRYOUTREG = 1) or unregistered

(CARRYOUTREG = 0). This output is to be connected only to

CARRYIN of adjacent DSP48A1 if multiple DSP blocks are used.

output CARRYOUTF;  //Carry out signal from post-adder/subtracter for use in the

FPGA logic. It is a copy of the CARRYOUT signal that can be

routed to the user logic.

//! Control Input Ports

input CLK;         //DSP clock

input [7 : 0] OPMODE; //Control input to select the arithmetic operations of the

DSP48A1 slice.

//! Clock Enable Input Ports

input CEA;        //Clock enable for the A port registers: (A0REG & A1REG).

input CEB;        //Clock enable for the B port registers: (B0REG & B1REG).

input CEC;        //Clock enable for the C port registers (CREG).

input CECARRYIN;  //Clock enable for the carry-in register (CYI) and the carry-

out register (CYO).

input CED;        //Clock enable for the D port register (DREG).

input CEM;        //Clock enable for the multiplier register (MREG).

input CEOPMODE;   //Clock enable for the opmode register (OPMODEREG).

input CEP;        //Clock enable for the P output port registers (PREG = 1).

//! Reset Input Ports: All the resets are active high reset. They are either sync or async depending on the parameter RSTTYPE.

input RSTA;       //Reset for the A registers: (A0REG & A1REG).

input RSTB;       //Reset for the B registers: (B0REG & B1REG).

input RSTC;       //Reset for the C registers (CREG).

input RSTCARRYIN; //Reset for the carry-in register (CYI) and the carry-out

register (CYO).

input RSTD;       //Reset for the D register (DREG).

input RSTM;       //Reset for the multiplier register (MREG).

input RSTOPMODE;  //Reset for the opmode register (OPMODEREG).

input RSTP;       //Reset for the P output registers (PREG = 1).

//! Cascade Ports:

output [17 : 0] BCOUT; //Cascade output for Port B.

input  [47 : 0] PCIN;  //Cascade input for Port P.

output [47 : 0] PCOUT; //Cascade output for Port P.

// !first stage

//regester -> A0REG, BOREG, DREG, CREG, OPMODEREG

wire [17 : 0] a0, b0, d;    // A0REG, BOREG, DREG

wire [47 : 0] c;            //CREG

wire [7 : 0] opmode_reg;    //OPMODEREG

wire [17 : 0] b_select;     // regster to select B or BCIN to store it in B0REG

based on the value of B_INPUT

assign b_select = (B_INPUT == "DIRECT") ? B : (B_INPUT == "CASCADE") ? BCIN

: 18'b0;

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(8)) OPMODE_REG (CLK, RSTOPMODE, CEOPMODE, OPMODEREG, OPMODE, opmode_reg);

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(18)) A0_STAGE (CLK, RSTA, CEA, A0REG, A, a0);

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(18)) B0_STAGE (CLK, RSTB, CEB, B0REG, b_select, b0);

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(48)) C0_STAGE (CLK, RSTC, CEC, CREG, C, c);

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(18)) D0_STAGE (CLK, RSTD, CED, DREG, D, d);

//! pre_adder/subtarcter

wire [17 : 0] out_pre;

assign out_pre = (opmode_reg[6] == 0) ? (d + b0) : (d - b0);

wire [17 : 0] out_mux_opmode_4;

assign out_mux_opmode_4 = (opmode_reg[4] == 1) ? out_pre : b0;

//! second stage

wire [17 : 0] a1, b1;

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(18)) B1_STAGE (CLK, RSTB, CEB, B1REG, out_mux_opmode_4, b1);

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(18)) A1_STAGE (CLK, RSTA, CEA, A1REG, a0, a1);

assign BCOUT = b1;

//! multiplication

wire [35 : 0] out_mul, mul_reg;

assign out_mul = b1 * a1;

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(36)) mul (CLK, RSTM, CEM, MREG, out_mul, mul_reg);

assign M = mul_reg;

//! x_multiplexer

wire [47 : 0] x_mux;

assign x_mux = (opmode_reg[1 : 0] == 2'b11) ? {d[11 : 0], a1[17 : 0], b1[17 : 0]} : (opmode_reg[1 : 0] == 2'b10) ?  PCOUT

: (opmode_reg[1 : 0] == 2'b01) ? {12'b0, mul_reg} : 0;

//! z_multiplexer

wire [47 : 0] z_mux;

assign z_mux = (opmode_reg[3 : 2] == 2'b11) ? c : (opmode_reg[3 : 2] == 2'b10) ? PCOUT : (opmode_reg[3 : 2] == 2'b01) ? PCIN : 0;

//!carry in

wire carry_select;

wire cin;

assign carry_select = (CARRYINSEL == "OPMODE5") ? opmode_reg[5]

: (CARRYINSEL == "CARRYIN") ? CARRYIN : 1'b0;

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(1)) CY1 (CLK, RSTCARRYIN, CECARRYIN, CARRYINREG, carry_select, cin);

//! post_adder/subtarcter

wire [47 : 0] out_post;

wire carry_out;

wire c_out;

assign {carry_out, out_post} = (opmode_reg[7] == 0) ? (z_mux + x_mux + cin)

: (z_mux - (x_mux + cin));

//! carry out

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(1)) CYO (CLK, RSTCARRYIN, CECARRYIN, CARRYOUTREG, carry_out, c_out);

assign CARRYOUT = c_out;

assign CARRYOUTF = CARRYOUT;

//! output p

wire [47 : 0] out_p;

REG_MUX #(.sync_type(RSTTYPE), .WIDTH(48)) OUT_P_REG (CLK, RSTP, CEP, PREG, out_post, out_p);

assign P = out_p;

assign PCOUT = P;

endmodule

//! The Pipeline_mux module

module REG_MUX(clk, rst, clk_enable, select, in, out);

parameter sync_type = "SYNC";

localparam synchronous = (sync_type == "SYNC");

localparam asynchronous = (sync_type == "ASYNC");

parameter WIDTH = 18;

input clk;

input rst;

input clk_enable;

input select;

input [WIDTH - 1 : 0] in;

output [WIDTH - 1 : 0] out;

reg [WIDTH - 1 : 0] d_ff;

generate

if(synchronous) begin

always@(posedge clk) begin

if(rst) begin

d_ff <= 0;

end

else if(clk_enable)begin

d_ff <= in;

end

end

end

else if(asynchronous) begin

always@(posedge clk or posedge rst) begin

if(rst) begin

d_ff <= 0;

end

else if(clk_enable)begin

d_ff <= in;

end

end

end

endgenerate

assign out = (select == 1) ? d_ff : in;

endmodule

# Testbench code

module dsp48a1_tb();

//! 1. The parameters will remain unchanged.

//! Control Input Ports

reg CLK;

reg [7 : 0] OPMODE;

//! Reset Input Ports: All the resets are active high reset. They are either sync or async depending on the parameter RSTTYPE.

reg RSTA, RSTB, RSTC, RSTCARRYIN, RSTD, RSTM, RSTOPMODE, RSTP;

//! Clock Enable Input Ports

reg CEA, CEB, CEC, CECARRYIN, CED, CEM, CEOPMODE, CEP;

// ! Data Ports:

reg [17 : 0] A, D, B, BCIN;

reg CARRYIN;

reg  [47 : 0] PCIN, C;

wire [17 : 0] BCOUT;

wire [47 : 0] PCOUT, P;

wire [35 : 0] M;

wire CARRYOUT, CARRYOUTF;

//! out expected

reg [17 : 0] BCOUT_expected;

reg [47 : 0] PCOUT_expected, P_expected;

reg [35 : 0] M_expected;

reg CARRYOUT_expected, CARRYOUTF_expected;

reg [47 : 0] past_Value_of_P;

reg past_value_of_CARRYOUT;

reg [48 : 0] result;

dsp48a1 uut(

CLK, RSTA, RSTB, RSTC, RSTD, RSTM, RSTOPMODE, RSTP, RSTCARRYIN,

CEA, CEB, CEC, CECARRYIN, CED, CEM, CEOPMODE, CEP, A, B, C, D,

BCIN, PCIN, CARRYIN, OPMODE,

CARRYOUT, CARRYOUTF, BCOUT, M, PCOUT, P

);

//! clock generation

initial begin

CLK = 0;

forever

#1 CLK = ~CLK;

end

//! 2. Stimulus Generation (Initial Block)

initial begin

//! initialize input

A = 0;  B = 0; C = 0;  D = 0;  BCIN = 0; CARRYIN = 0;

OPMODE = 0; CEA = 0; CEB = 0; CEC = 0; CECARRYIN = 0;

CED = 0; CEM = 0; CEOPMODE = 0; CEP = 0; PCIN = 0;

BCOUT_expected = 0; PCOUT_expected = 0; M_expected = 0; P_expected = 0;

CARRYOUT_expected = 0; CARRYOUTF_expected = 0;

past_Value_of_P = 0; result = 0;

//! 2.1. Verify Reset Operation

//Assert all active-high reset signals by setting them to 1.

RSTA = 1; RSTB = 1; RSTC = 1; RSTCARRYIN = 1;

RSTD = 1; RSTM = 1; RSTOPMODE = 1; RSTP = 1;

//Drive remaining inputs with arbitrary (random) values.

A = $random; B = $random; C = $random; D = $random; BCIN = $random;

CARRYIN = $random; OPMODE = $random; PCIN = $random;

CEA = $random; CEB = $random; CEC = $random; CECARRYIN = $random;

CED = $random; CEM =$random; CEOPMODE = $random; CEP = $random;

//Wait for the negative edge of the clock.

@(negedge CLK);

//Add a condition for self-checking to verify that all outputs are zero.

BCOUT_expected = 0; PCOUT_expected = 0; M_expected = 0; P_expected = 0;

CARRYOUT_expected = 0; CARRYOUTF_expected = 0;

if(BCOUT_expected != BCOUT)

$display("ERROR IN RESET BCOUNT");

else

$display("RESET BCOUNT pass");

if(PCOUT_expected != PCOUT)

$display("ERROR IN RESET PCOUT");

else

$display("RESET PCOUT pass");

if(M_expected != M)

$display("ERROR IN RESET M");

else

$display("RESET M pass");

if(P_expected != P)

$display("ERROR IN RESET P");

else

$display("RESET P pass");

if(CARRYOUT_expected != CARRYOUT)

$display("ERROR IN RESET CARRYOUT");

else

$display("RESET CARRYOUT pass");

if(CARRYOUTF_expected != CARRYOUTF)

$display("ERROR IN RESET CARRYOUTF");

else

$display("RESET CARRYOUTF pass");

//Deassert all reset signals and assert all clock enable signals to validate

the functionality of the subsequent DSP paths.

RSTA = 0; RSTB = 0; RSTC = 0; RSTCARRYIN = 0; RSTD = 0; RSTM = 0;

RSTOPMODE = 0; RSTP = 0; CEA = 1; CEB = 1; CEC = 1; CECARRYIN = 1;

CED = 1; CEM = 1; CEOPMODE = 1; CEP = 1;

//! 2.2. Verify DSP Path 1

//corresponding to OPMODE = 8'b11011101.

OPMODE = 8'b11011101;

// Apply the following input values: A = 20, B = 10, C = 350, and D = 25.

A = 20; B = 10; C = 350; D = 25;

//Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

BCIN = $random; CARRYIN = $random; PCIN = $random;

//The expected outputs are: BCOUT = 'hf, M = 'h12c, P = PCOUT = 'h32, and

CARRYOUT = CARRYOUTF = 0.

BCOUT_expected = 'hf; PCOUT_expected = 'h32; M_expected = 'h12c;

P_expected = 'h32;  CARRYOUT_expected = 0; CARRYOUTF_expected = 0;

//Wait for four negative clock edges, as the data propagates through four

flip-flops (DREG, B1REG, MREG, and PREG)

repeat(4) @(negedge CLK);

//Add a condition for self-checking to verify that the design outputs with

the expected outputs.

if(BCOUT_expected != BCOUT)

$display("ERROR IN Path 1 BCOUNT");

else

$display("Path 1 BCOUNT pass");

if(PCOUT_expected != PCOUT)

$display("ERROR IN Path 1 PCOUT");

else

$display("Path 1 PCOUT pass");

if(M_expected != M)

$display("ERROR IN Path 1 M");

else

$display("Path 1 M pass");

if(P_expected != P)

$display("ERROR IN Path 1 P");

else

$display("Path 1 P pass");

if(CARRYOUT_expected != CARRYOUT)

$display("ERROR IN Path 1 CARRYOUT");

else

$display("Path 1 CARRYOUT pass");

if(CARRYOUTF_expected != CARRYOUTF)

$display("ERROR IN Path 1 CARRYOUTF");

else

$display("Path 1 CARRYOUTF pass");

//! 2.3. Verify DSP Path 2

// corresponding to OPMODE = 8'b00010000.

OPMODE = 8'b00010000;

//Apply the following input values: A = 20, B = 10, C = 350, and D = 25.

A = 20; B = 10; C = 350; D = 25;

//Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

BCIN = $random; CARRYIN = $random; PCIN = $random;

// The expected outputs are: BCOUT = 'h23, M = 'h2bc, P = PCOUT = 0,and

CARRYOUT = CARRYOUTF = 0.

BCOUT_expected = 'h23; PCOUT_expected = 'h0; M_expected = 'h2bc;

P_expected = 'h0;  CARRYOUT_expected = 0; CARRYOUTF_expected = 0;

//Wait for three negative edges

repeat(3) @(negedge CLK);

//Add a condition for self-checking to verify that the design outputs with

the expected outputs.

if(BCOUT_expected != BCOUT)

$display("ERROR IN Path 2 BCOUNT");

else

$display("Path 2 BCOUNT pass");

if(PCOUT_expected != PCOUT)

$display("ERROR IN Path 2 PCOUT");

else

$display("Path 2 PCOUT pass");

if(M_expected != M)

$display("ERROR IN Path 2 M");

else

$display("Path 2 M pass");

if(P_expected != P)

$display("ERROR IN Path 2 P");

else

$display("Path 2 P pass");

if(CARRYOUT_expected != CARRYOUT)

$display("ERROR IN Path 2 CARRYOUT");

else

$display("Path 2 CARRYOUT pass");

if(CARRYOUTF_expected != CARRYOUTF)

$display("ERROR IN Path 2 CARRYOUTF");

else

$display("Path 2 CARRYOUTF pass");

//! 2.4. Verify DSP Path 3

//corresponding to OPMODE = 8'b00001010

past_Value_of_P = P;

past_value_of_CARRYOUT = CARRYOUT;

OPMODE = 8'b00001010;

//Apply the following input values: A = 20, B = 10, C = 350, and D = 25.

A = 20; B = 10; C = 350; D = 25;

//Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

BCIN = $random; CARRYIN = $random; PCIN = $random;

//The expected outputs are: BCOUT = 'ha, M = 'hc8, P = PCOUT which is the

past Value of P,and CARRYOUT = CARRYOUTF which is the past value of CARRYOUT.

BCOUT_expected = 'ha;

result = 2 * past_Value_of_P;

PCOUT_expected = result[47 : 0];

M_expected = 'hc8;

P_expected = result[47 : 0];

CARRYOUT_expected = result[48];

CARRYOUTF_expected = result[48];

//Wait for three negative edges

repeat(3) @(negedge CLK);

//Add a condition for self-checking to verify that the design outputs with

the expected outputs.

if(BCOUT_expected != BCOUT)

$display("ERROR IN Path 3 BCOUNT");

else

$display("Path 3 BCOUNT pass");

if(PCOUT_expected != PCOUT)

$display("ERROR IN Path 3 PCOUT");

else

$display("Path 3 PCOUT pass");

if(M_expected != M)

$display("ERROR IN Path 3 M");

else

$display("Path 3 M pass");

if(P_expected != P)

$display("ERROR IN Path 3 P");

else

$display("Path 3 P pass");

if(CARRYOUT_expected != CARRYOUT)

$display("ERROR IN Path 3 CARRYOUT");

else

$display("Path 3 CARRYOUT pass");

if(CARRYOUTF_expected != CARRYOUTF)

$display("ERROR IN Path 3 CARRYOUTF");

else

$display("Path 3 CARRYOUTF pass");

//! 2.5. Verify DSP Path 4

//corresponding to OPMODE = 8'b10100111.

OPMODE = 8'b10100111;

//Apply the following input values: A=5, B=6, C=350, D=25 and PCIN=3000

A = 5; B = 6; C = 350; D = 25; PCIN = 3000;

// Drive BCIN, and CARRYIN with arbitrary (random) values.

BCIN = $random; CARRYIN = $random;

//The expected outputs are: BCOUT = 'h6, M = 'h1e,

P = PCOUT ='hfe6fffec0bb1, and CARRYOUT = CARRYOUTF = 1.

BCOUT_expected = 'h6; PCOUT_expected = 'hfe6fffec0bb1; M_expected = 'h1e;

P_expected = 'hfe6fffec0bb1; CARRYOUT_expected = 1; CARRYOUTF_expected = 1;

//Wait for three negative edges

repeat(3) @(negedge CLK);

//Add a condition for self-checking to verify that the design outputs with

the expected outputs.

if(BCOUT_expected != BCOUT)

$display("ERROR IN Path 4 BCOUNT");

else

$display("Path 4 BCOUNT pass");

if(PCOUT_expected != PCOUT)

$display("ERROR IN Path 4 PCOUT");

else

$display("Path 4 PCOUT pass");

if(M_expected != M)

$display("ERROR IN Path 4 M");

else

$display("Path 4 M pass");

if(P_expected != P)

$display("ERROR IN Path 4 P");

else

$display("Path 4 P pass");

if(CARRYOUT_expected != CARRYOUT)

$display("ERROR IN Path 4 CARRYOUT");

else

$display("Path 4 CARRYOUT pass");

if(CARRYOUTF_expected != CARRYOUTF)

$display("ERROR IN Path 4 CARRYOUTF");

else

$display("Path 4 CARRYOUTF pass");

$stop;

end

endmodule

# Do file

# QuestaSim Snippets

## Verify Reset Operation

Verify Reset Operation

• Assert all active-high reset signals by setting them to 1.

• Drive remaining inputs with arbitrary (random) values.

• Wait for the negative edge of the clock.

• Add a condition for self-checking to verify that all outputs are

zero.

• De assert all reset signals and assert all clock enable signals to

validate the functionality of the subsequent DSP paths.

### Waveform

### Transcript

## Verify DSP Path 1

Verify DSP Path 1

• This test path as shown in figure 1 evaluates the pre-subtractor,

allowing it to propagate and post-subtractor stages by enabling

the multiplier output to route through Mux X and the C-port

through Mux Z, corresponding to OPMODE = 8'b11011101.

• Apply the following input values:

A = 20, B = 10, C = 350, and D = 25.

• Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

• Wait for four negative clock edges, as the data propagates

through four flip-flops (DREG, B1REG, MREG, and PREG), as

illustrated in Figure (1).

• The expected outputs are: BCOUT = 'hf, M = 'h12c,

P = PCOUT = 'h32, and CARRYOUT = CARRYOUTF = 0.

• Add a condition for self-checking to verify that the design

outputs with the expected outputs.

Figure 1 Path 1 Data Flow

### Waveform path 1

### Transcript path 1

## Verify DSP Path 2

Verify DSP Path 2

• This test path evaluates the pre-addition allowing it to

propagate and post-addition stages by enabling the zeros to route

through Mux X and Mux Z, corresponding to OPMODE = 8'b00010000.

• Apply the following input values:

A = 20, B = 10, C = 350, and D = 25.

• Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

• Wait for three negative edges, as the data propagates through

three flip-flops (DREG, B1REG, MREG) as longest path and also

pass to PREG in parallel, as illustrated in Figure (2).

• The expected outputs are: BCOUT = 'h23, M = 'h2bc, P = PCOUT = 0,

and CARRYOUT = CARRYOUTF = 0.

• Add a condition for self-checking to verify that the design

outputs with the expected outputs.

Figure 2 Path 2 Data Flow

### Waveform path 2

### Transcript path 2

## Verify DSP Path 3

Verify DSP Path 3

• This test path has no pre-addition/subtractor (doesn’t allow it

to propagate) and allows post-addition stages by enabling the P

feedback to route through Mux X and Mux Z, corresponding to

OPMODE = 8'b00001010.

• Apply the following input values:

A = 20, B = 10, C = 350, and D = 25.

• Drive BCIN, PCIN, and CARRYIN with arbitrary (random) values.

• Wait for three negative edges, as the data propagates through

three flip-flops (B1REG, MREG, PREG) as longest path and also

pass to PREG in parallel, as illustrated in Figure (3).

• The expected outputs are: BCOUT = 'ha, M = 'hc8, P = PCOUT which

is the past Value of P, and CARRYOUT = CARRYOUTF which is the pat

value of CARRYOUT.

• Add a condition for self-checking to verify that the design

outputs with the expected outputs.

Figure 3 Path 3 Data Flow

### Waveform path 3

### Transcript path 3

## Verify DSP Path 4

Verify DSP Path 4

• This test path has no pre-addition/subtractor (doesn’t allow it

to propagate) and allows post-subtractor stages by enabling D:A:B

Concatenated to route through Mux X and PCIN to Mux Z,

corresponding to OPMODE = 8'b10100111.

• Apply the following input values:

A = 5, B = 6, C = 350, D = 25 and PCIN = 3000

• Drive BCIN, and CARRYIN with arbitrary (random) values.

• Wait for three negative edges, as the data propagates through

three flip-flops (B1REG, MREG, PREG) as longest path as

illustrated in Figure (4).

• The expected outputs are: BCOUT = 'h6, M = 'h1e,

P = PCOUT = 'hfe6fffec0bb1, and CARRYOUT = CARRYOUTF = 1.
• Add a condition for self-checking to verify that the design

outputs with the expected outputs.

Figure 4 Path 4 Data Flow

### Waveform path 4

### Transcript path 4

# Constraint File

## Clock signal

set_property -dict { PACKAGE_PIN W5   IOSTANDARD LVCMOS33 } [get_ports CLK]

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports CLK]

## Configuration options, can be used for all designs

set_property CONFIG_VOLTAGE 3.3 [current_design]

set_property CFGBVS VCCO [current_design]

## SPI configuration mode options for QSPI boot, can be used for all designs

set_property BITSTREAM.GENERAL.COMPRESS TRUE [current_design]

set_property BITSTREAM.CONFIG.CONFIGRATE 33 [current_design]

set_property CONFIG_MODE SPIx4 [current_design]

# Elaboration

## Messages

## Schematic snippets

# Synthesis

## Messages

## Utilization report

## Timing report

## Schematic snippets

# Implementation

## Messages

## Utilization report

## Timing report

## Device snippets

# Linting

## Snippets showing no errors